ANLEITUNG ZUR RHYTHMUS-SOFTWARE FÜR ELKA X/C 1000/2000

ALLGEMEIN

-------------------------------------------------------------------------------------------------------------------------------

Sie haben mit der RHYTHMUS-Software ein - von vielen ELKA-Besitzern lang erwartetes Update erworben. Ziel unserer Arbeit war es, die vorhandenen Rhythmen neu zu überarbeiten und, vor

allem, neue Rhythmen zu schaffen, die auf Ihrem Keyboard seither nicht vorhanden waren. Die Rhythmen befinden sich in diversen Dateien auf der Diskette. Jede Datei beinhaltet 10 Rhythmen, die nach dem Kopiervorgang auf die Taster 1 - 10 Ihrer Rhythmussektion gelegt werden. Um welche Rhythmen es sich jeweils handelt, können Sie aus dem beiliegenden Verzeichnis entnehmen.

GRUNDSÄTZLICHES

-------------------------------------------------------------------------------------------------------------------------------

Jeder Rhythmus besteht aus einem eintaktigem INTRO,

dem zwei-taktigem GRUNDRHYTHMUS mit Arrangement und Bass,

ein eintaktiges FILL IN, welches über die Taster "FILL IN 1" mit Bass und

"FILL IN 2" mit Bass und Arrangement abgerufen werden kann,

sowie ein ein- bis zweitaktiges Ending (1.Takt mit Bass u. Arr., 2.Takt quasi mit Schlussakkord). Die Funtionen eines BREAK bleiben bestehen.

Bei den Orgeln ELKA X1000, C1000, C2000 ist es technisch leider nicht möglich, zu einem "Intro", "Ending", oder "Fill In"  ein spezifisches Arrangement oder eine Baßspur zu programmieren.

KURZANLEITUNG ZUM EINLADEN DER DATEIEN AUS "RHYTHMS_" + "EXCLUSIV"

-------------------------------------------------------------------------------------------------------------------------------

1. - Diskette einlegen

2. - Taster EDIT betätigen

3. - F4 (Files)

4. - NEXT  PAGE bis "Dir" erscheint

5. - F3 (Dir) es erscheint nun der Name der Diskette (zB RHYTHMS1)

sowie die freie Speicherkapazität der Diskette in kByte.

6. - mit F5 (OK) bestätigen

7. - mit F1 (Select) die gewünschte Datei aufrufen

8. - F2 (Copy->)

9. - F5 (OK)

10.- F5 (OK) Display: "Disk is busy.....please wait" (ca 30 sek.)

11.- Der Kopiervorgang ist abgeschlossen, sobald wieder das Aus-

gangsdisplay erscheint. Taster NORMAL verläßt die Programmier

ebene. Die Taster 1-10 sind jetzt mit den neuen Rhythmen be-

legt.

Zum Einladen der nächsten Datei von der Diskette brauch das In-

haltsverzeichnis der Diskette nicht mehr aufgerufen werden.

Also: - EDIT - F4 - NEXT PAGE - F1 - F2 - F5 - F5 -

TIPS

-------------------------------------------------------------------------------------------------------------------------------

Jeder der neuen Rhythmen ist mit einem spezifischen Tempo programmiert worden. Nach dem Kopiervorgang kann es sein, daß - da sich Rhythmusdateien nur komplett abspeichern lassen - die Tempi Ihrer Grungrhythmen sich nun verändert haben (Je nachdem welche Tempi Sie vorher eingestellt hatten). Tip: Stellen Sie nach dem Kopiervorgang alle Tempi (mittels TEMPO SET) und Taster (ROOT LOGIC etc) so ein, wie Sie es möchten (auch den neuen Rhythmen können sie mit TEMPO SET ein neues, Ihrem Geschmack entsprechendes Grundtempo

geben). Nehmen Sie die Diskette (zB RHYTHMS1) aus dem Diskettenlaufwerk heraus und legen Sie eine leere, formatierte Diskette ein. Jetzt kopieren Sie die Rhythmusdatei in der von Ihnen geänderten Fassung auf die leere Diskette.

Die Vorteile liegen klar auf der Hand: Erstens haben Sie nun eine Sicherheitskopie der Rhythmus-

diskette, Zweitens haben Sie die Tempi und Tasterstellungen so, wie Sie sie brauchen und Drittens können Sie nun auch die Dateien der anderen Rhythmus-Disketten (zB "RHYTHMS2" od. "EXCLUSIV") verändert auf diese eine Diskette ablegen (Diskettenwechsel entfällt).

---------------------Viel Spaß !!!------------------------------------------------------------------------------------------

INSTRUCTIONS FOR RHYTHMUS-SOFTWARE FOR ELKA X/C 1000/2000

GENERAL

-------------------------------------------------------------------------------------------------------------------------------

You have purchased with the ELKA RHYTHMS-Software a long expected update. Our objective was, to revise the existing Rhythmen and above all to create new rhythms, which was not present on your keyboard before. This rhythm-files are saved in several file-banks. Each file-bank includes 10 rhythms, which will be stored to the keys 1 to 10 of your rhythm-section after the copy-proceeding. Which rhythms are in which file-bank you can see in the below list.

FUNDAMENTAL

-------------------------------------------------------------------------------------------------------------------------------

Each Rhythm has a one-bar INTRO,

a two-bar BASICRHYTHM with Arrangement and Bass,

a one-bar FILL IN, which can be called with the key "FILL IN 1" (with Bass) and

"FILL IN 2" (with Bass and Arrangement),

as soon as an one- to two-bar Ending (1. bar with Bass and Arrangement, 2.bar as it where with “final chord”). The functions of a BREAK continues.

At the Multikeyboards ELKA X1000, C1000, C2000 it is technical impossible, to program for an "Intro", "Ending", or "Fill In"  a specific Arrangement or an own bass-track.

INSTRUCTIONS TO LOAD THE FILES OF  "RHYTHMS_" + "EXCLUSIV"

-------------------------------------------------------------------------------------------------------------------------------

1. - 	Insert floppy disc

2. - 	Press key EDIT

3. - 	F4 (Files)

4. - 	NEXT  PAGE until "Dir" appears

5. - 	F3 (Dir) now it appears the name of the disc (e.g. RHYTHMS1)

as soon as the free capacity of the disc in kbyte

6. - 	confirm with F5 (OK)

7. - 	with F1 (Select) select the requested file.

8. - 	F2 (Copy->)

9. - 	F5 (OK)

10.- 	F5 (OK) display says: "Disk is busy.....please wait" (ca 30 sec.)

11.- 	The copy proceeding has finished, if you see the start-display-informations. Key NORMAL leaves the program-level. The keys 1-10 are stored now with the new Rhythms.

To load the next file from disk you must not call the directory once more.

Only push the following keys:

- EDIT - F4 - NEXT PAGE - F1 - F2 - F5 - F5 -

TIPS

-------------------------------------------------------------------------------------------------------------------------------

Each of the new rhythms has been programmed with a specific tempo. After the copy-proceeding it may be that the speed of your basic-rhythms has been changed, because it is just possible to save the settings complete (depending on which speed was selected before).

Tip: After the copy-proceeding set all rhythms-speed (with TEMPO SET) and key (ROOT LOGIC etc) like you want (also the new rhythms can changed in tempo with TEMPO SET in a new tempo, which you prefer). Remove the disc (e.g. RHYTHMS1) from the discdrive and insert an empty, formatted (of course formatted on ELKA), floppy disc. Now copy the rhythmsfile in the changed version to the empty disc.

The befits are obvious: First you have a security-copy of your rhythms-disc, second you have settings of tempo and speed adjusted so as you need it and third you can also store the other filebanks on the same method to this disk (You have then just one disc for all your rhythms)

---------------------Have Fun !!!------------------------------------------------------------------------------------------

| ** RHYTHMS1 ** 80 Styles in 8 x 10 Bänken |  |  |  |  |
| --- | --- | --- | --- | --- |
| Nr | STANDRD1 | STANDRD2 | DANCE 1 | DANCE 2 |
| 01 | Waltz 1 | Blues | Beat 1 | Dnce-Beat 1 |
| 02 | Waltz 2 | Slow-Fox | Beat 2 | Dnce-Beat 2 |
| 03 | Slow-Waltz | Ballad | Disco 1 | Dnce-Beat 3 |
| 04 | US-March | Swing-Shuffle | Disco 2 | Dnce-Beat 4 |
| 05 | Marsch-Fox | Quickstep | Pop 1 | Disco-Fox 1 |
| 06 | Paso-Doble 1 | Charleston | Pop 2 | Disco-Fox 2 |
| 07 | Paso-Doble 2 | Jive 1 | Funk | Disco-Shuffle |
| 08 | Tango | Jive 2 | Techno | Dc-Sl.Rock 1 |
| 09 | Polka 1 | Foxtrott | New Wave | Dc-Sl.Rock 2 |
| 10 | Polka 2 | Big Band | Heavy | Dance-Ballad |
| Nr | LATIN 1 | COUNTRY | LTSSWING | VOLKSMUS |
| 01 | Rhumba | Bluegrass 1 | Rock 'n Roll 1 | Eger-Polka 1 |
| 02 | Beguine | Bluegrass2 | Rock 'n Roll 2 | Eger-Polka 2 |
| 03 | Mambo | Sqre-Dance | Shuffle 1 | Eger-Walzr 1 |
| 04 | Calypso | Go-West | Shuffle 2 | Eger-Walzr 2 |
| 05 | Bossa-Nova | Cntry-Ball. 1 | Twist | Steirisch |
| 06 | Cha-Cha | Cntry-Ball. 2 | Dixie | Steir. Walzer |
| 07 | Samba 1 | Country 1 | Ragtime | Rheinländer |
| 08 | Samba 2 | Country 2 | Take Five | Eg.-Marsch 1 |
| 09 | Reggae | Western | Slow-Swing | Eg.-Marsch 2 |
| 10 | Latin | Arizona | Skiffle | Resi-Beat |

| ** RHYTHMS2 ** 80 Styles in 8 x 10 Bänken |  |  |  |  |
| --- | --- | --- | --- | --- |
| Nr | STANDRD3 | DANCE 3 | DANCE 4 | SPECIAL |
| 01 | Waltz 3 | Dance-Beat 5 | Disco 5 | Tarantella |
| 02 | Musette-Wlz. | Dance-Beat 6 | Disco 6 | Sirtaki |
| 03 | Slow-Waltz 2 | Dance-Beat 7 | Marmor-Beat | Afro |
| 04 | Tango 2 | Dance-Beat 8 | Disco-Fox 3 | Marchina |
| 05 | Tango 3 | Disco 3 | Disco-Fox 4 | Baion |
| 06 | Slow-Rock 1 | Disco 4 | Clap-March | Guaracha |
| 07 | Slow-Rock 2 | Pop 3 | Pop 5 | Bamboleo |
| 08 | Rhumba 2 | Pop 4 | Pop 6 | Trinis Tune |
| 09 | Beguine 2 | Rock-Ballad 1 | Pop 7 | Boot-Mix 1 |
| 10 | Condor | Rock-Ballad 2 | Pop 8 | Boot-Mix 2 |
| Nr | LATIN 2 | CLASSIC | OLDBASIC | VOLKHITS |
| 01 | Limbo | Bolero 1 | Boogie 1 | Polka 3 |
| 02 | Salsa 1 | Bolero 2 | Boogie 2 | Polka 4 |
| 03 | Salsa 2 | Wiener Wlz 1 | Blues 2 | Steirisch 2 |
| 04 | Reggae 2 | Wiener Wlz 2 | Ballad 2 | Steir.Walz. 2 |
| 05 | Rhumba 3 | Granada | Shuffle 3 | Marsch 3 |
| 06 | Samba 3 | Klass. Marsch | Shuffle 4 | Marsch 4 |
| 07 | Latin-Disco | 6/8 March 1 | Rock 'n Roll 3 | Tanz 1 |
| 08 | Cha-Cha 2 | 6/8 March 2 | Twist 2 | Tanz 2 |
| 09 | Tequilla Prt 1 | Rondo | New York 1 | Fast Waltz |
| 10 | Tequilla Prt 2 | Serenade | New York 2 | Steirisch-Slow |