GEBRAUCHSANWEISUNG FÜR **SOUNDDAT**

INHALTSVERZEICHNIS UND DATENMENGEN

TRANSPOS.PRG    2 kB                VOL_1_EK.DAT

RAMSOUND.PRG    3 kB                     bis

RSOUNDS_.DAT   11 kB                VOL_14EK.DAT   jew. 11 kB

ROM_____.DAT   11 kB                65_99_04.SND   2 kB

__NULL__.DAT   11 kB                1_99_SND.SND   2 kB

GRUNDSÄTZLICHES

Bevor Sie nun mit dem Einladen der Dateien und Programme von der Diskette in den Arbeitsspeicher der X1000/C1000/C2000 beginnen, sollten Sie bedenken, daß - auch bei Geräten mit eingebauter Speichererweiterung - eventuell die freie Kapazität des Arbeitsspeichers zu klein sein könnte. Deshalb schaffen Sie bitte zunächst Platz, indem Sie Demo- und Sequenzersongs, die bekanntlich recht viel Speicherplatz in Anspruch nehmen, aus dem internen Speicher löschen. (Natürlich erst, nachdem Sie sie vorher auf Diskette gesichert haben).

ANLEITUNG ZUM EINLADEN DER DATEIEN AUS **SOUNDDAT**

1.	Diskette **SOUNDDAT** einlegen, Taster EDIT betätigen,

2.	F4 (Files), dann "NEXT PAGE" bis über F3 "Dir" erscheint,

3.	F3 (Dir), Display anschließend mit F5 (ok) bestätigen,

4.	mit F1 (select) gew. Datei von Disk. wählen (zB.TRANSPOS.PRG)

5.	F2 (copy->), dann F5 (ok), wieder F5 (ok)

Die Datei wird nun in den Arbeitsspeicher übertragen. Wiederholen Sie nun die Schritte 	und 5, um die restlichen Dateien ebenfalls in den Speicher zu laden. Nun kann es sein, 	daß bei Instrumenten ohne eingebauter Speichererweiterung der interner Speicher vorzeitig 	voll ist, obwohl Sie vorher durch Löschen bestimmter Sequenzersongs Platz gemacht haben. 	Das ist nicht weiter schlimm, denn wie Sie später sehen werden, können Sie aus den 448 	Sounds die "Besten" auswählen und in der __NULL.DAT mit 64 Sounds ablegen, neu 	benennen und archivieren.

TRANSPOSE- UND RAMSOUND-PROGRAMME

Nachdem Sie die Programme und Dateien in den internen Speicher der ELKA X1000/C1000/C2000 übertragen haben, gehen Sie wie folgt vor:

TRANSPOS.PRG:

1.	mittels Taster EDIT verlassen Sie die Kopierebene

2.	betätigen Sie nun F5 (User Progs.)

3.	Wählen Sie nun mit den Funktionstastern (F1-F5) "TRANSP"

4.	im Display erscheint: "Oktav-Transponierung"

5.	F1 "weiter"

6.	Sie können nun mit den Funktionstastern F1 - F4 die einzelnen Bereiche Ihres Instrumentes 	(Obermanual, Untermanual Links, Untermanual Rechts (bei Keyboard-Split) und Bass) je 	Geschmack um eine Oktave nach oben oder nach unten transponieren.

7.	mit Taster F5 (Ende) verlassen Sie die Transpose-Ebene.

-1-

-2-

RAMSOUND.PRG:

1.	Nachdem Sie die Transp.-Ebene verlassen haben, wählen Sie nun mit den Funktionstastern 	(F1-F5) "RAMSND"

2.	Im Display erscheint: Installation von RAM-Sounds

3.	betätigen Sie nun F1 (RAM)

4.	Display:    Sound File: .........DAT

suchen  setzen   -  -  copy Pg Ende

F1      F2      F3     F4     F5

5.	wählen Sie nun mit F1 (suchen) eine der Sounddateien aus (z. B. VOL_1_EK.DAT)

6.	mit F2 (setzen) schreiben Sie nun die Datei in die Sektion ORCHESTRA 4

7.	Sie können nun mit den Tastern 1-32 der "Presets Bank" aus "Orchestra 4" die 32 neuen 	Sounds der Datei (Liste anbei) ausprobieren.

8.	Wiederholen Sie die Schritte 5 und 6, um die anderen Sounddateien ebenfalls 	ausprobieren und testen zu können.

Aus der ganzen Vielfalt der neuen Sounds können Sie nun die, die Ihnen am besten gefallen in der sog."__NULL__.DAT ablegen.

Dazu gehen Sie bitte wie folgt vor:

1.	Wählen Sie mit F1 (suchen) eine der Sounddateien aus, in der sich die gewünschten Sounds 	befinden

2.	Mit F2 (setzen) die gewählte Datei installieren

3.	Taster F4 (copy Pg) betätigen

4.	mit F1 (File) die Datei anwählen, in die kopiert werden soll. in unserem Fall wäre das: 	__NULL__.DAT

5.	Mit der Tastatur der "PresetsBank"(Orch.4) gewünschten Sound wählen (zB "DX-	Piano2")

6.	mit F3 oder +/- Taster die Soundnummer der NULL.Datei wählen, auf die der entsprechende 	Sound kopiert werden soll

7.	Taster F4 (copy) betätigen. Der Sound ist nun in der NULL.Dat

8.	Wiederholen Sie nun die Schritte 5 - 7 , um die anderen gewünschten Sounds aus der 	Sounddatei in die NULL.Datei zu kopieren.

9.	Betätigen Sie nun Taster "Normal", um in die Ausgangsebene zu gelangen

10.	Wiederholen Sie nun die Schritte 1 - 9 bis die 64 freien Speicherplätze der NULL.Datei 	mit Ihren gewünschten Sounds überschrieben sind.

Diese Datei(__NULL__.DAT) kann nun umbenannt und archiviert werden

1.	Mit F5 (Ende) User Prog - Ebene verlassen

2.	F4 (Files)

3.	F1 (select) bis: "memory: __NULL__.DAT"

4.	evtll. mit F4 (prot.) Schreibschutz entfernen

5.	F5 (Name),  dann neuen Namen eingeben.

6.	ggf. auf Diskette abspeichern

7.	mit Taster "EDIT" Ebene verlassen, und dann Taster F5 (UserProg) betätigen um evt. die 	nächste Sounddatei zusammenzustellen.

Achtung: Wollen Sie nun Ihre ursprünglichen Sounds wiederhaben, betätigen Sie - nachdem Sie das RAMSND Programm wieder aufgerufen haben - F2 (ROM) .

Nun sind wieder Ihre Standard-ROM-Sounds installiert.

******************************************************************

Wir wünschen Ihnen nun viel Freude mit Ihrer Sound- und Transpose-

software

1992

INSTRUCTIONS FOR **SOUNDDAT**

Content and file-capacity

TRANSPOS.PRG    2 kB                VOL_1_EK.DAT

RAMSOUND.PRG    3 kB                     bis

RSOUNDS_.DAT   11 kB                VOL_14EK.DAT   jew. 11 kB

ROM_____.DAT   11 kB                65_99_04.SND   2 kB

__NULL__.DAT   11 kB                1_99_SND.SND   2 kB

GENERAL

Before you begin to load the files and programs into the RAM-memory of your ELKA X1000/C1000/C2000, you should consider that the free memory-capacity could be to small - also at equipment with inbuilt RAM-extension. Therefore please make first room for the files and programs, by deleting demo and sequencingsongs of your memory, which needs regular much memory-space (Of course after you made a security-copy of that files)

INSTRUCTION TO LOAD THE FILES FROM **SOUNDDAT**

1.	Insert disc **SOUNDDAT**, then push key EDIT,

2.	F4 (Files), then "NEXT PAGE" til you read above F3 "Dir",

3.	F3 (Dir), Display next confirm with F5 (ok),

4.	with F1 (select) select requested file from disc (e.g.TRANSPOS.PRG)

5.	F2 (copy->), then F5 (ok), and F5 (ok) again.

The file will now be transfered into the RAM-space.

Repeat now this steps and step 5 to load all the other files into the RAM-space.

Now it could be that at equipment without inbuilt memoryextension, the memory space is full before all files has been loaded, although you made room with deleting files before. That’s no tragedy, because, as you can read later below, you are able to choose your favorite sounds from the 448 sounds and save it to your own private soundbank with 64 sounds.

TRANSPOSE- AND RAMSOUND-PROGRAMS

After you have transfered the programs and files into your intern space memory of your ELKA X1000/C1000/C2000 you do following:

TRANSPOS.PRG:

1.	Leave the copy-level with key EDIT 
2.	push now F5 (User Progs.)

3.	Select now with the function-keys (F1-F5) "TRANSP"

4.	in display appears: "Oktav-Transponierung"

5.	F1 "go on"

6.	You are now able to transpose with the function-keys F1 - F4 the single zones of your instrument (upper manual, lower manual left, lower manual right (if keyboard-split) and bass) one octave up or down

7.	wit key F5 (Ende) leave the transpose-level.

-3-

-4-

RAMSOUND.PRG:

1.	After you have leaved the Transpose-level, select now with the function-keys (F1-F5) "RAMSND"

2.	In display appears: Installation von RAM-Sounds

3.	confirm now with: F1 (RAM)

4.	Display:    Sound File: .........DAT

suchen  setzen   -  -  copy Pg Ende

F1      F2      F3     F4     F5

5.	select now with F1 (suchen) one of the soundfiles (e.g. VOL_1_EK.DAT)

6.	with F2 (setzen) you’ll write the sounds into the section ORCHESTRA 4

7.	You vcan now test the 32 new sounds with the keys 1-32 of the "Presets Bank" of "Orchestra 4" (see also the soundlist below)

8.	Repeat the steps 5 and 6, to test and load the other soundbanks

From this range of new sounds you are now able to create your own soundbank, by saving the sounds into the file: "NULL.DAT”.

For it you should do following steps:

1.	Select with F1 (suchen) one of the soundfiles, which contents the requested sound

2.	With F2 (setzen) install the requested file

3.	Press key F4 (copy Pg)

4.	with F1 (File) select that bank, where you like to copy to. In that case it would be the bank: ”NULL__.DAT”

5.	With the keypannel of the "PresetsBank"(Orch.4) select the requested sound (e.g. "DX-	Piano2")

6.	with F3 or +/- keys select the soundnumber of NULL.DAT, to which the selected sound shall be copied.

7.	Press key F4 (copy). This sound is now placed in NULL.Dat

8.	Repeat the steps  5 - 7, to copy the other requested sounds from the usual soundbanks to “NULL.Dat”.

9.	Press now the key "Normal", to arrive the starting level

10.	Repeat now the steps 1 - 9 until all 64 free memoryspaces of the „NULL.Dat“ has been overwrited with your requested sounds.

This private soundbank (NULL.DAT) can now renamed and archived.

1.	With key F5 (Ende) leave User Prog - level

2.	F4 (Files)

3.	F1 (select) until: "memory: NULL.DAT"

4.	If necessary remove write-protection with F4 (prot.)

5.	F5 (Name),  then enter new name.

6.	if necessary save to floppy-disc

7.	leave the level with key "EDIT", and then push key F5 (UserProg) if you like to compile the   next soundfile

ATTENTION: Do you like to get back your standard sounds, please push - after you called up the RAMSND Programm again - - F2 (ROM) .

Now your standard-ROM-sounds are installed again.

******************************************************************

We hope you enjoy now your Sound- und Transpose-Software

1992

| CONTENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BANK 1 | BANK 2 | BANK 3 | BANK 4 | BANK 5 | BANK 6 | BANK 7 |
| 01 | CX Piano 1 | Jump 1 | Fl.Brass 2 | Slap Bass | Snare 1C 3 | Basschord | Desaster 1 |
| 02 | DX Piano 1 | Jump 2 | Piano 1 | Plikpluk 1 | Snare 2 C 3 | Bassynth 1 | Desaster 2 |
| 03 | DX Piano 2 | Wiredrops | Zitheraal | Fender | Snare 3 E 3 | Analogsyn | Synt.piano |
| 04 | Brass 1 | Plikpluk 1 | Dig. Brass 1 | Fender low | Snare 4 C 3 | Star Track | Rhodes 1 |
| 05 | Panflute | Bass & Flag | Dig. Brass 2 | Bass & Flag | Bluesharp | Hallneed2 | E-Harfe 1 |
| 06 | Brass 2 | Met. Piano 2 | Cosyiano 1 | Fretless | Toms | Solopete 1 | E-Harfe 2 |
| 07 | Marimba | Met. Brass | Desaster 3 | Contrabass | Dschungel | Hallneed 1 | Wurlitzer |
| 08 | French Horns | E.P. & Brass | Trombone | Soft Bass | Wurlitzer | Dyn.Clav. | Space Str. |
| 09 | Harp | Ch. Bass | Syn.clav. 1 | Perc. Synth | Strings 2 | PPG Metal | Karimba 1 |
| 10 | Moogsolo | Slap Bass | FM Bass | Plikpluk 2 | Milkybell | Fender Dyn. | Vangelis 2 |
| 11 | Moogsolo 2 | Moogsolo 3 | FM Solo | CO 2 | Le Traffik | Rockpiano | Fantasia 4 |
| 12 | Slow Horns | Cosyiano 1 | Piano 2 | Perc.Synth | Orbitwalk | Fender MK | Hallneed 4 |
| 13 | Moogbass | Trumpets 1 | Steel Drum | Analogsyn. | Agogo 1 | Strings 2 | Spacesia |
| 14 | Sitar | Haw. Guitar | Cosyiano 3 | PPG Voice | Hi Hat | CP 70 M | Flute |
| 15 | Bell Piano 3 | Clavichord | Cosyiano 4 | PPG Voice 2 | Slapper | Toots | Glockenspiel |
| 16 | DX Piano 4 | Fender Rh. 2 | Flagelee | Ruler Bass | Tabla 2 | Steinwayb. | Zungenpiano |
| 17 | Harp Piano 1 | Zither | Fl Brass | Hammond 1 | Teppich | Vibrapart | Harppiano |
| 18 | Tänzerin | Brat Gitr. | Piano 3 | CO 2 1 | Strings | Steinwayb. 2 | Fruehling |
| 19 | Fender Rhd. | Space Strngs | Pianoforte | PPG Vibes | Teppich 2 | Vibraphon | Bellorgan |
| 20 | E-Piano 2 | Spacebell | Braz Sect 1 | Strings 1 | Analog | Steinwayc. | HarpBello |
| 21 | Ballerbaz | Spieluhr | Dig. Brass 3 | St.Drums 2 | Perc. Synth | Part.Piano | Fantasia 2 |
| 22 | Wurlitzer | Vocal 2 | Discobass | Fat Slap 1 | Bassdrum 1 | Marim.Seq. | Fantasia 3 |
| 23 | Klingelei | Trumpets 2 | Plikpluk 1 | Bass 4 | Bass 2 | Glasvoice | Strat. |
| 24 | Strings 1 | Vibes 3 | Synstrato | Contrabass 3 | PPG2Vibes | Rockpiano 2 | Syntpiano |
| 25 | Lionel H. | ZZ Top 2 | Vollenw. 1 | Saite 1 | Congas | Rockpiano 3 | Syn.clav. 1 |
| 26 | Akk. Guitar | Oboe-Fag. | FM Bass 1 | Slap Bass 3 | Marimba | Ethno | Spectrum 4 |
| 27 | Whistlers | Oboe-Fag. 2 | Gläser 2 | Bass 2 | Congawooh | DawnBell | Spectrum 5 |
| 28 | Bell Piano 1 | Clavichord 2 | Space Vibe | Bass 3 | Cowbell E 3 | Milchglas | Spectrum 3 |
| 29 | Wecker | Jahrmarkt | Gläser 1 | Fretless 3 | Bongos | Voice fem. | Spectrum 2 |
| 30 | Akkordeon 1 | Brass 5 | Space Vibe 2 | Contrabass 2 | Woodblock | Glas Piano | Spectrum 1 |
| 31 | Akkordeon 2 | Tuba | 3-4-5 mal | Fretless 2 | Tabla | Clav. FM | Fantasia 1 |
| 32 | Akkordeon 3 | Fretless | Discobass 2 | Saite 2 | Bimmel | Glas Piano 2 | Rhodes 2 |
|  | BANK 8 | BANK 9 | BANK 10 | BANK 11 | BANK 12 | BANK 13 | BANK 14 |
| 01 | Vibrafono | Sax Sopran | Phase Pad 1 | Piano 5 | Flute & Guit. | E-Piano 6 | CP Piano |
| 02 | Claves | SpaceHarp | Dolby Pad 0 | Piano 6 | Nylon Guit. | Organ 5 | Rhodes 1 |
| 03 | Paradise 1 | Picked 1 | Git. Syn. 3 | Fun Piano | Syn Brass 7 | Organ 6 | Rhodes 2 |
| 04 | Hit Me 3 | Sax Ensemb. | Tin. Clav 2 | Viola Orch. | Syn Orch. | Organ 7 | Rhodes 3 |
| 05 | Marimba 5 | Bell Strings 2 | Funky 01 | E.P. & Strings | Harpsich. 1 | Sound 1 | Vibes 2 |
| 06 | Trom Mangi | Syn.Strat. 2 | Atmosph. 03 | Syn. & Bells | Strings 6 | Sound 2 | Marimba 1 |
| 07 | Holz Bass | Nylon 1 | Atmosph. 02 | Brass 4 | Strings 7 | Syn Brass 7 | Calimba |
| 08 | Shamus Th 3 | SquareSyn. | Atmosph. 05 | Brass 5 | Strings 8 | Trump.Fan | Strings 12 |
| 09 | Zischquok | Trombone 2 | Atmosph. 04 | Rev. Brass | Strings 9 | Fuzz Clav. | Org.Smith |
| 10 | Electric 3 | Gamelan 1 | Atmosph. 01 | Met. Brass | Clarinet | Slapp Bass 1 | Org. Prok. |
| 11 | Violin 1 | Arco Bass 1 | Analogie 6 | E.P. & Brass | Flute & Vib. | Fretless | HawaiiGuitar |
| 12 | Prophet | Pastorius | Analogie 1 | Syn. Brass 3 | Flute & Str. | Strings 10 | Choir 1 |
| 13 | Snip E2-A2 | Spectrum | Analogie 2 | Syn. Brass 4 | Organ 3 | Strings 11 | Choir 2 |
| 14 | Bass Guit. | Brillant 1 | Analogie 3 | Syn. Brass 5 | Organ 4 | Syn.Guitar 1 | Choir 3 |
| 15 | Shamus Th 2 | Blowrimba | Analogie 8 | Syn. Brass 6 | Pipe 2 | Syn.Guitar 2 | Pipe 3 |
| 16 | Strings | Brt. Piano | Analogie 5 | Brass & Str. | Pipe 3 | Syn.Marim. | Pipe 4 |
| 17 | Hit Me 5 | Bell Str. 1 | Analogie 4 | Guitar 4 | Akkordeon 2 | Perc.Wood | Pipe 5 |
| 18 | Hit Me 1 | Cosyiano 5 | Analogie 11 | Guitar 5 | Syn Marim. 1 | Vibes 1 | Pipe 6 |
| 19 | Sq. Mallet | Xylo Pad 1 | Analogie 7 | Guit. & Synt. | Toys | Vangelis | Trombone 1 |
| 20 | Seq. Synth. | Spacyx | Bell Syn. 3 | Vocal 2 | Syn Clav. 3 | Crescendo | Trombone 2 |
| 21 | Perc.Synth. | Bottle Mall. | X Brass 00 | Ch. Bass | Syn. Lead 1 | Fairy Step | Trumpet |
| 22 | Moogbass 2 | Glas Piano 1 | Cool Vibes | Dyna Bass | An. Synth. 8 | Double | Clarinet 1 |
| 23 | Ruler Bass | Hammer 1 | PPG Metal | Organ 3 | An. Synth. 9 | Space Lab | Sax 1 |
| 24 | Castaniet | Sax C1000a | PVC Harp 1 | Pipe Org. 2 | An. Synth. 10 | Dist. Gtr. | Jazz Flute |
| 25 | Fat Metal 2 | Mallet 1 | EP 12Strg. | An. Synth. 1 | Samp. Voice | Invert | Whistle |
| 26 | Solo Synth | Arco Bass 2 | Dolby Pad 1 | An. Synth. 2 | Syn Perc. | Syn. Drum | Violin 1 |
| 27 | Moogbass | Synvoice 1 | Moogbass 4 | An. Synth. 3 | Syn Bass 2 | Perc. Synth. | Oboe 1 |
| 28 | Pizzicat 1 | Soft Piano | Moogbass 3 | An. Synth. 4 | Syn Bass 3 | Mand. & Str. | Harmonica |
| 29 | Glasharp | Oooh 2 | Funky 00 | An. Synth. 5 | Machinery | Ghosts | Akkord. 3 |
| 30 | Pizzicat 2 | Oooh 1 | Git. Syn. 4 | An. Synth. 6 | Train | Syn Space | Akkord. 4 |
| 31 | Metal&Ooh | Sax 2a | Atmosph. 06 | An. Synth. 7 | Space Guitar | Ambience | Syn. Lead 1 |
| 32 | Fat Metal 1 | Gamelan 2 | Analogie 10 | Synth & Wind | El. Drums | Amb. 2001 | Space Str. |